Road Anomaly Detection Using YOLOv11s and Edge Deployment

1. Introduction

Road infrastructure quality directly impacts transportation safety, vehicle maintenance cost, and mobility efficiency. This project proposes a real-time Road Anomaly Detection System using a custom-trained YOLOv11s object detection model deployed on both GPU systems and edge devices like Raspberry Pi. Road infrastructure quality directly impacts transportation safety, vehicle maintenance cost, and public mobility efficiency. Manual road inspection is labor-intensive, slow, and prone to human error. With advancements in computer vision and embedded AI, automated road monitoring systems can provide real-time detection of road anomalies such as potholes, cracks, and uneven road surfaces.

This project proposes a real-time Road Anomaly Detection System using a custom-trained YOLOv11s object detection model deployed on both high-performance GPU systems and edge devices like Raspberry Pi. The system detects multiple road-related entities including:

- Road damages (potholes, cracks, surface defects)

- Speed bumps

- Unsurfaced roads

- Vehicles

- Pedestrians

The system is designed to be lightweight, real-time capable, and suitable for deployment on mobile platforms such as drones or vehicles

- Objectives

The primary objectives of this project are:

- Develop a custom dataset for road anomaly detection.

- Train a deep learning-based object detection model.

- Optimize the model for real-time inference.

- Deploy the system on embedded hardware.

- Achieve high detection accuracy with low latency.

3. System Overview

The system consists of data collection, model training, optimization, deployment, and real-time detection pipeline.

3.1 Data Collection & Annotation

The performance of any deep learning-based object detection system heavily depends on the quality and diversity of the dataset used for training. For this project, a custom Road Anomaly Detection (RAD) dataset was created to ensure that the model learns real-world road conditions and anomaly variations.

The dataset was collected from multiple sources to improve variability and generalization capability. These sources include:

Real-world road footage captured from mobile cameras, publicly available road monitoring datasets, online road infrastructure imagery drone-based aerial road inspection footage. The collected images represent diverse environmental and operational conditions such as: Different lighting conditions (daytime, evening, shadows) Various weather conditions (sunny, cloudy, wet roads), Urban and rural road environments, Different camera angles distances and Varying traffic density. This diversity ensures that the trained model remains robust and performs reliably in practical deployment scenarios.

Dataset Organization

The collected dataset was structured following the YOLO object detection format to support efficient training and validation. The dataset was divided into three subsets:

  - Training Set (70%): Used to train the model parameters

  - Validation Set (20%): Used for tuning hyper-parameters and monitoring training performance

  - Test Set (10%): Used for final unbiased performance evaluation

3.2 Model Training & Validation

Model Selection

For the Road Anomaly Detection system, the YOLOv11s (You Only Look Once – Version 11 Small) architecture was selected due to its balance between detection accuracy and computational efficiency. YOLO-based detectors are single-stage object detection models that perform classification and localization simultaneously, making them highly suitable for real-time applications and edge deployment.

The YOLOv11s variant provides:

- Lightweight architecture suitable for embedded devices

- High inference speed

- Multi-scale feature extraction

- Improved detection performance for small and medium-sized objects

Transfer Learning Approach

Instead of training the model from scratch, transfer learning was applied using pretrained YOLOv11 weights. This approach enables faster convergence and improved detection performance by leveraging feature representations learned from large-scale datasets.

The pretrained weights were fine-tuned using the custom RAD dataset to adapt the model to road anomaly detection tasks.

Training Environment

Model training was conducted using the Ultralytics YOLO framework in a GPU-accelerated environment.

Hardware Configuration:

- GPU: NVIDIA GeForce RTX 3060 Laptop GPU (6GB VRAM)

- CPU: Multi-core laptop processor

- Framework: Ultralytics YOLO

- Programming Language: Python

Training Configuration

The training process was configured to optimize detection accuracy while preventing over-fitting.

| Parameter | Value |
| --- | --- |
| Model | YOLOv11s |
| Training Epochs | 100 |
| Input Image Resolution | 768 × 768 |
| Batch Processing | GPU Optimized |
| Optimization Strategy | Stochastic Gradient Descent based optimizer |
| Loss Functions | Classification Loss, Bounding Box Regression Loss, Objectness Loss |

During training, images were resized and normalized before being passed into the neural network. Data augmentation techniques such as scaling, flipping, and color variations were applied to improve model robustness.

Training Process

The training pipeline consisted of the following stages:

- Dataset Loading: Images and corresponding annotations were loaded and preprocessed.

- Data Augmentation: Random transformations were applied to increase dataset variability.

- Forward Propagation: Input images were processed through the YOLOv11s architecture.

- Loss Computation: Model predictions were compared with ground truth labels.

- Backpropagation: Gradients were calculated and used to update network weights.

- Model Checkpointing: Best-performing weights were automatically saved based

The training process required approximately 30 hours to complete.

Validation Strategy

A dedicated validation dataset was used to evaluate model performance after each training epoch. Validation helps monitor model generalization and detect overfitting.

Performance was evaluated using standard object detection metrics including:

- Precision

- Recall

- Mean Average Precision (mAP)

The best performing model weights were saved as best.pt. The validation results obtained during training are summarized below:

| Class | Precision | Recall | mAP@0.5 | mAP@0.5:0.95 |
| --- | --- | --- | --- | --- |
| Overall | 0.753 | 0.716 | 0.753 | 0.438 |
| Heavy Vehicle | 0.921 | 0.976 | 0.979 | 0.764 |
| Light Vehicle | 0.874 | 0.892 | 0.964 | 0.689 |
| Pedestrian | 0.838 | 0.903 | 0.910 | 0.561 |
| RoadDamages | 0.691 | 0.393 | 0.487 | 0.203 |
| Speed-Bump | 0.645 | 0.729 | 0.746 | 0.411 |
| UnsurfaceRoad | 0.569 | 0.684 | 0.617 | 0.366 |

Training Observations

- Vehicle and pedestrian classes achieved high accuracy due to abundant training samples.

- Road damage detection remains challenging because of irregular shapes and background similarity.

- Higher input resolution significantly improved detection of small anomalies.

- Transfer learning reduced training time and improved convergence stability.

3.3 Model Optimization

The trained YOLOv11s PyTorch model (best.pt) was converted into lightweight inference formats to improve compatibility with edge devices.

NCNN Conversion

NCNN is a high-performance neural network inference framework optimized for ARM-based processors commonly used in embedded systems. Converting the model into NCNN format provides several advantages:

- Reduced model size

- Faster inference execution on CPU-based devices

- Lower memory consumption

- Improved compatibility with Raspberry Pi architecture

NCNN enables efficient parallel computation and optimized matrix operations, making it suitable for real-time object detection on low-power devices.

TensorFlow Lite (TFLite) Conversion

TensorFlow Lite is designed for mobile and embedded machine learning applications. The YOLOv11s model was also converted into TFLite format to enable:

- Hardware acceleration support

- Efficient runtime memory management

- Compatibility with quantized inference

- Faster loading and execution times

TFLite models provide flexibility for deployment across different embedded AI platforms.

3.3.2 INT8 Quantization

Quantization is a technique used to reduce the precision of numerical computations within a neural network. In this project, INT8 quantization was applied to convert floating-point model weights into 8-bit integer representations.

Benefits of INT8 Quantization

- Significant reduction in model size

- Reduced memory bandwidth requirements

- Lower computational complexity

- Reduced power consumption

- Faster inference speed on CPU-based hardware

- Although quantization introduces slight precision loss, experimental results showed that the model retained strong detection performance while achieving substantial improvements in runtime efficiency.

3.3.3 FP16 Precision Optimization

Half-precision floating point (FP16) optimization was applied during model export and inference on GPU-supported environments.

Advantages of FP16 Optimization

- Reduced GPU memory usage

- Increased computational throughput

- Faster matrix operations

- Improved real-time performance

FP16 allows efficient parallel processing on GPUs while maintaining acceptable numerical precision. This optimization is particularly useful during high-resolution inference or multi-frame processing.

3.3.4 Multi-thread CPU Optimization

Embedded devices such as Raspberry Pi have limited CPU resources. By default, deep learning libraries may spawn excessive computational threads, leading to performance instability and increased latency. To prevent CPU resource saturation, thread control mechanisms were implemented using environment variables:

- OMP_NUM_THREADS

- OPENBLAS_NUM_THREADS

- MKL_NUM_THREADS

- Restricting thread usage ensures:

- Stable inference performance

- Reduced context switching overhead

- Lower CPU utilization

- Improved system responsiveness

This optimization is critical for maintaining consistent real-time detection performance.

3.3.5 Input Resolution Tuning

Input image resolution directly affects model accuracy and computational complexity. Multiple resolutions were evaluated to identify an optimal trade-off between detection performance and inference speed.

Resolution Analysis

| Resolution | Performance | Speed | Observation |
| --- | --- | --- | --- |
| 320 × 320 | Fast | Low accuracy | Misses small anomalies |
| 512 × 512 | Balanced | Moderate speed | Good trade-off |
| 768 × 768 | High accuracy | Higher latency | Best anomaly detection |

Higher resolutions improve detection of small and irregular road defects but increase processing time. Therefore, resolution tuning was performed based on deployment requirements and hardware capability.

3.3.6 Overall Optimization Impact

The combined optimization techniques resulted in:

- Reduced model size

- Improved inference speed

- Lower memory usage

- Stable real-time detection performance

- Enhanced suitability for embedded deployment

These optimizations enabled successful deployment of the Road Anomaly Detection system on low-power edge devices without significant degradation in detection accuracy.

5. Methodology

The methodology of the Road Anomaly Detection system describes the complete workflow followed to design, develop, train, optimize, and deploy the object detection model. The proposed system uses deep learning-based computer vision techniques to automatically detect road anomalies in real-time.

The methodology consists of several sequential stages, including dataset preparation, model training, validation, optimization, and deployment.

- Data Acquisition and Preparation

The first step in the methodology involved collecting a diverse dataset representing real-world road environments. The dataset included images captured from road monitoring systems, publicly available sources, and manually collected road imagery.

Each image was annotated using bounding boxes to mark different objects such as: Road damages (potholes, cracks), Speed bumps, Unsurfaced roads, Vehicles, Pedestrians

The dataset was then divided into three subsets:

- Training dataset for model learning

- Validation dataset for performance monitoring

- Test dataset for final performance evaluation

To improve generalization capability, preprocessing and augmentation techniques such as resizing, flipping, and brightness adjustments were applied.

2. Model Selection

The YOLOv11s architecture was selected as the core detection model. YOLO is a single-stage object detection algorithm that performs object classification and localization simultaneously, enabling high-speed detection suitable for real-time applications.

The YOLOv11s variant was specifically chosen because it offers: Lightweight architecture, Fast inference speed, Efficient multi-scale object detection and Suitability for edge deployment

3. Transfer Learning and Training

Transfer learning was used to accelerate model training and improve detection accuracy. Pretrained YOLOv11 weights were fine-tuned using the custom road anomaly dataset.

The training process involved:

- Feeding annotated images into the neural network.

- Extracting image features using convolutional layers.

- Predicting bounding boxes and class labels.

- Computing losses between predicted and actual annotations.

- Updating model weights using backpropagation.

Training was conducted for 100 epochs using GPU acceleration to ensure faster convergence and improved performance.

4. Model Validation and Performance Evaluation

Model performance was evaluated after each training epoch using a separate validation dataset. Standard object detection metrics were used, including: Precision, Recall and Mean Average Precision (mAP). These metrics ensured that the model maintained balanced detection accuracy and minimized false detections.

After training, the best-performing model was selected based on validation results and tested using unseen data to confirm real-world performance.

5. Model Optimization

To enable real-time deployment on embedded systems, multiple optimization techniques were applied:

- Conversion of the trained model into NCNN and TensorFlow Lite formats

- INT8 quantization to reduce computational complexity

- FP16 precision optimization for faster GPU inference

- Multi-thread CPU optimization for stable execution

- Input resolution tuning for performance balancing

These optimizations significantly reduced model size and inference latency while maintaining detection accuracy.

6. Real-Time Detection Pipeline

The optimized model was integrated into a real-time detection pipeline. The pipeline processes live video or camera input through the following steps:

- Capturing video frames

- Preprocessing frames to match model input size

- Running model inference

- Generating bounding boxes and class predictions

- Rendering detection results on the output frame

- Displaying or storing processed video

7. Edge Deployment

The optimized model was deployed on Raspberry Pi 4 to demonstrate practical implementation on low-power hardware. The deployment included:

Environment setup using Python virtual environments

OpenCV-based video streaming, NCNN or TFLite runtime inference and Thread management for stable processing

This allowed the system to perform real-time road anomaly detection in embedded environments.

8. System Validation in Real-World Conditions

The deployed system was tested using live camera input and recorded road videos. The system successfully detected road anomalies and surrounding objects under varying environmental conditions, validating its real-world applicability.

6. Challenges

During the development and deployment of the Road Anomaly Detection system, several technical and practical challenges were encountered. These challenges influenced model performance, deployment feasibility, and system reliability.

6.1 Dataset Imbalance

One of the major challenges was the imbalance in dataset class distribution. Vehicle and pedestrian samples were significantly higher compared to road anomaly classes such as potholes and cracks.

This imbalance caused the model to learn dominant features associated with frequently occurring classes, which sometimes reduced detection accuracy for rare anomaly classes. Road damages are naturally less frequent and highly irregular, making it difficult to collect sufficient labeled samples.

To mitigate this issue, careful dataset curation and augmentation techniques were applied. However, increasing anomaly-specific samples remains essential for further performance improvement.

6.2 Small Pothole Detection Difficulty

Detecting small and irregularly shaped potholes presented a significant challenge. Road anomalies often have:

Non-uniform shapes, Low contrast with road surfaces, Texture similarity with surrounding areas, Partial occlusion due to vehicles or shadows.

Small defects are particularly difficult to detect because object detection models tend to prioritize larger and more prominent features. Higher input image resolutions improved detection accuracy but increased computational requirements.

6.3 Edge Device Computation Limitations

Deploying deep learning models on Raspberry Pi introduced hardware-related constraints such as:

Limited CPU processing power, Restricted memory availability, Absence of dedicated GPU acceleration, Power consumption limitations These constraints required careful model optimization, including quantization, lightweight architecture selection, and thread management, to maintain stable inference performance.

6.4 Real-Time Latency Constraints

Achieving real-time detection while maintaining high accuracy required balancing input resolution, model complexity, and computational efficiency.

Higher-resolution inputs improved anomaly detection but increased inference time. Real-time processing demanded optimization of frame capture, preprocessing, inference, and visualization pipelines to minimize latency.

Ensuring stable frame rates under varying environmental conditions remained a key challenge during system deployment.

7. Future Improvements

- Dataset expansion

- Segmentation model integration

- GPS anomaly mapping

- Temporal video tracking

- Autonomous drone deployment

8. Conclusion

This project presents a real-time Road Anomaly Detection system based on deep learning and edge computing technologies. A custom dataset was developed, and a YOLOv11s object detection model was successfully trained and optimized for real-world deployment. The system demonstrates strong detection performance across multiple road-related classes while maintaining efficient inference on resource-constrained hardware such as Raspberry Pi. Optimization techniques including model conversion, quantization, and resolution tuning enabled practical deployment without significant loss of accuracy.

The proposed system highlights the potential of combining computer vision with embedded AI to improve road safety, infrastructure monitoring, and automated inspection processes. With further dataset expansion and advanced model integration, the system can be extended into large-scale smart transportation and autonomous monitoring applications.